Avenza for iPhone users

Avenza Maps is a mobile map app that allows you to download maps for offline use on your iOS or Android smartphone or tablet. The app uses your device's GPS and no internet or cell phone service is required when you are in the field, but it is important to download the program and all maps ahead of time using a wifi connection.

The Free version of Avenza can hold up to three specialty maps at once. The maps created for Site Stewards are all specialty maps. Many other maps can be purchased and downloaded from the map store without limit. The Avenza map created for your site is a georeferenced pdf file.  It is a good idea to review some of the YouTube videos on Avenza to get a better idea of what the application can do.

In order to use Avenza, first download the app

- Download Avenza Maps app from the App Store (Apple) or Google Play

- Allow Avenza to use your gps  - turn on location data.

- In Avenza go to Settings and make following changes :

Unit of Measurement – Change Coordinates Format to first option under UTM-WGS 84.  Make sure Use Map Coordinate Setting is OFF.

Map View – Use True North – set to ON, Also turn Keep Screen ON

Display – change Crosshair Color to Red (Red is much easier to see when you are out in the field)

How to get a map from an email into Avenza program - connect to wifi

- Open email with Avenza map pdf attachment

- Click on pdf

- In upper right corner select Box with up arrow – a list of icons will appear

- Scroll left… select Avenza Icon…if you don’t see the Avenza Icon, select … More Box

- Scroll up until you see Avenza Icon, select.  Your map will load. Click on it to open.

How to pin a point on an Avenza Map where you are standing

- Open base map of the area you are in, you will notice that you are the blue dot on the map

- Before placing a pin, wait 10-15 seconds for the satellite signals to pinpoint your location more accurately. If your blue dot is bouncing around, wait a little longer.  If it does not ‘settle’ your GPS point accuracy may not be within 4 meters, take the point anyway.

- If you are standing on the point you want to pin, select the arrow in the upper right hand corner, tap the arrow until the bullseye covers the blue dot. Please note, tapping the arrow cycles through 3 settings.  IMPORTANT: For an accurate GPS location the bullseye must be over the blue dot. Do not try to move the blue dot to the bullseye, this will not give you an accurate location.

- Tap the Pin Icon (in lower right corner)

- Name your Placemark, ie:  PS01 (Photo station 01), F01, ART01, etc.

- Tap Submit

- The location is now pinned on a layer of your base map.

- To see the GPS location of the point you pinned, tap the pin – this will open “edit placemark”.  Scroll down to Location – click arrow.  You will see something like 11 5557797 E  3679644 N. 5557797 is your Easting number and 3679644 is your Northing number.  To close this screen, click back and close.

- Tap the arrow again to remove the bullseye.

- A note about photos.  Do not take photos from within the Avenza program. It’s best to take them using your phone’s camera, another app (Solocator) or digital camera.

- To Delete a pin, click the pin to open the Edit Placemark screen and tap the garbage can at the bottom right. If you are sure, click Delete.

- Important Note: Make sure that you write down GPS coordinates in addition to pinning on Avenza Maps.  Electronics can fail, so having written backup is very important.

- If you tap the Pin before you have a bullseye – just cancel/do not save the placemark and try again.

Layers – When you place a pin on your map it is stored on a layer, not on the base map.

Layers are associated with the base map you have open. Be sure to save layers before deleting any base maps.  If you delete a base map, you will also delete the layer and all saved pins will be lost. Layers are not stored in the on-line store.

To see all your pinned locations, open your base map.  Click the stack of 3 sheets in the lower right corner.  You will see “layer” and # of objects and Active.   Touch Layer and your pinned locations will be listed.  Clicking on a location will take you to that location on the base map.

We recommend saving your layers on your computer to ensure that you have a backup.

Locking and Unlocking a Map Layer

When working with a map it is possible to inadvertently move a pin unless the layer is locked. We suggest when you have completed creating pins such as for your photo stations, the layer should be locked.

- With your map open the Map Layers screen by clicking the stack of 3 sheets in lower right corner

- Choose your layer and click the three dots to the right

- Choose Edit then slide the Lock layer bar on or off as desired

Instructions for saving and sending Avenza Layers

Saving layers ensures that the points you pinned are preserved and can be retrieved.  It’s a good backup practice, as layers will be lost if a map is deleted or your smart phone fails.  The first thing you should do is rename the layer.

- Open the Avenza Map

- Select the stack of 3 items in lower right corner

- Select the 3 dots on the layer you want to rename

- Edit, change the Name, go Back

To Send the Layer – open the map associated with the layer

- Click the stack of 3 items in lower right corner.

- Select the 3 dots

- Click Export – check to make sure that Custom 1 layer is selected.  The number of features should match your photo log for this project.

- Click EXPORT in upper right corner.

- Select Mail

- Send it to yourself and then move the file into a folder.  You can also mail it to someone else at the same time.

Importing a layer

Open the Map Layers screen by clicking the stack of 3 sheets in lower right corner

- Click the “ +” in the lower right corner.

- Choose Import layers

- Choose Cloud storage or device and navigate to the location where you stored your layers

- The file will automatically download into the Map layer

Importing a layer from an email

- Open the email

- Click on the kml file

- Click the box with up arrow in lower left corner

- Select Avenza Maps. Layer will be imported.

A note about Easting and Northing #s

The numbers are in meters based on a grid.  If you move east, the easting number will increase.  If you move north the northing number will increase – one meter at a time.  This is useful if you have a GPS location that you are trying to get to.

Additional iphone Settings that help in the field

Turn off Auto-Dimming: Open Settings > Accessibility > Display & Text Size and tap the Auto-Brightness toggle. Then go to Settings> Display & Brightness, and turn off Automatic, True Tone and slide Brightness slide bar to right.

Turn off Night Shift: Open Settings > Display & Brightness > Night Shift and tap the Scheduled toggle.

Your iPhone display will also dim when Low Power Mode activates due to the battery running low.

Resources

Avenza Getting Started:  https://support.avenzamaps.com/hc/en-us/articles/5050417116052-Getting-Started

Avenza Tutorials:   https://support.avenzamaps.com/hc/en-us/sections/5524662246676-Tutorials

Questions?  Corrections?  Suggestions?  Please email Pam at pwiedenkeller@gmail.com